Tian Huang

UNSW-COMP-9331

Nov 10th, 2025

LAB 4

Exercise 1:

Question 1:

Server: IP 128.119.245.12, Port 80(HTTP)

Client: IP 192.168.1.102, Port 1161(source)

Question 2:

The TCP contains HTTP POST command is in Frame 4, Seq Number 1 and Len 565

Question 3:

Part A:

Part B & C:

| Frame | Sent Time | Seq | Len | ACK | RTT | Est. RTT |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | 0.026477 | 1 | 565 | 0.053937 | 0.02746 | 0.02746 |
| 5 | 0.041737 | 566 | 1460 | 0.077294 | 0.03556 | 0.02846 |
| 7 | 0.054026 | 2026 | 1460 | 0.124885 | 0.07086 | 0.03359 |
| 8 | 0.054690 | 3486 | 1460 | 0.169118 | 0.11443 | 0.04310 |
| 10 | 0.077405 | 4946 | 1460 | 0.217299 | 0.13989 | 0.05495 |
| 11 | 0.078157 | 6406 | 1460 | 0.267802 | 0.18965 | 0.07168 |

Part D:

The first segment is 565 bytes, the others are 1460 bytes each.

Question 4:

Min advertise windows size 5840 bytes

It does not throttle the sender

Question 5:

No

Question 6:

1460 bytes of data would be receiver typically acknowledge in an ACK

Some ACK acknowledges 2920 bytes, which is 2 times of 1460, there is delayed ACK

Question 7:

Calculation: (171kB * 1000B/kB)/ (5.651s * 1000ms/s) = 30260 bytes/s

= 242kbps

Wireshark data: 241kbps

Exercise 2:

Question 1:

SEQ 2818463618, Frame 295

Question 2:

SEQ 1247095790 Frame 296

ACK 2818463619

It comes from Question 1, the init SEQ + 1

Question 3:

SEQ 2818463619

ACK 1247095791

No

Question 4:

The client(10.9.16.201) done the active close.

Frame 304 client send FIN, ACK to server

It is 4 segment, FIN->ACK->FIN->ACK

Question 5:

Client -> Server: 33 bytes

Server -> Client: 40 bytes

Data acknowledge corresponds to the difference of the final ACK and init SEQ